DEPARTMENT OF COMPUTER SCIENCE & ENGINEERING
OPERATING SYSTEMS (CS3001) — MODULE 1

ACADEMIC PROJECT REPORT

Operating System Visualization & Simulation Platform
An Interactive Architectural Workbench for Direct Memory Access (DMA) and Multicore vs. Multiprocessor Organizations

| Course Title: Operating Systems (CS3001) Academic Module: Module 1 (Architecture & Overview) Date of Submission: October 24, 2026 | Submitted To: Prof. Shivashish Gour Dept. of Computer Science & Engineering Submitted By (Project Team): • Nikhil (DMA Subsystem & Bus Engine) • Sakshi (Multicore/MP & Fault Engine) |
| --- | --- |

Project Team & Individual Contributions

The engineering work was partitioned into two distinct micro-architectural domains:

| Team Member | Module & Primary Responsibilities | Key Code & Test Deliverables |
| --- | --- | --- |
| Nikhil | Topic 5: Direct Memory Access (DMA) & Bus Arbitration • 8237 DMA Finite State Machine & HOLD/HLDA Handshake • 20-Cycle Logic Analyzer Oscilloscope Waveform • Crossbar Highway Arbiter & Contention Turnstile Queue | DmaSimulator.jsx UnifiedSystemLab.jsx Test Case 1 Verification |
| Sakshi | Topic 6: Multicore vs. Multiprocessor Organizations • 2/4/8 Core Die Scaler & Thermal Hotspot Engine (48°-89°C) • SMP vs. Distributed RAM & L3 Cache Ring Bus Model • Fault Injection Engine (Die Overheat vs. Socket Pull) | MultiArchitectureComparator.jsx OsEvolutionTimeMachine.jsx Test Cases 2 & 3 |

1. Executive Summary & Abstract

Traditional classroom instruction in Operating Systems frequently encounters pedagogical bottlenecks when communicating low-level hardware concurrency. Micro-architectural mechanisms—such as the hardware bus release handshake between the CPU and the DMA controller, cycle-level processor stalls, bus contention turnstiles, and inter-core cache snooping latencies—occur at nanosecond scales within closed integrated circuits. Consequently, students frequently memorize comparison tables (e.g., Slide 20) without internalizing the physical trade-offs.

To resolve this educational gap, the OS-ArchLab Platform was designed, engineered, and verified. Built upon modern reactive frontend systems (Node.js, React 19, Vite) and rendered with hardware-accelerated Cyberpunk telemetry animations, the platform models discrete system clock pulses, animated electrical bus lines, live hardware registers, DRAM memory cell allocations, 20-cycle logic analyzer waveforms, and runtime fault injection (Thermal Die Overheating and Physical CPU Socket Disconnection).

2. Curriculum Mapping & Syllabus Alignment

The simulation modules map directly to the official syllabus lecture slides of Module 1: Operating System Overview:

| Syllabus Topic | Lecture Slide Ref. | Conceptual Core | Visual Module |
| --- | --- | --- | --- |
| Topic 5: Direct Memory Access | Slide 13 | DMA Definition, 'Station Master' Analogy, 8237 Controller | DmaSimulator.jsx |
| Topic 5: DMA Working Protocol | Slide 14 | 6-Step Transfer Protocol, HOLD/HLDA Handshake, IRQ Interrupt | DmaSimulator.jsx |
| Topic 5: DMA Modes of Operation | Slide 15 | Burst Mode, Cycle Stealing Mode, Transparent Mode | DmaSimulator.jsx |
| Topic 6: Multicore Organization | Slides 16–17 | Single Silicon Die, Shared L3 Cache, Thermal Density, Inter-Core Latency | MultiArchitectureComparator.jsx |
| Topic 6: Multiprocessor Organization | Slides 18–19 | Multi-Socket Motherboard, System Bus Arbiter, SMP vs. Distributed RAM, Fault Isolation | MultiArchitectureComparator.jsx |
| Hardware Bus Crossroads | Slides 18–19 | Memory Bus Contention, Arbiter Turnstile, Starvation Prevention | UnifiedSystemLab.jsx |
| OS Historical Generations | Slides 23, 35–39 | 1st to 5th Gen OS, Programmed I/O vs. Batch vs. DMA vs. Multicore | OsEvolutionTimeMachine.jsx |

3. Theoretical Foundations & Mathematical Cycle Models

3.1 Direct Memory Access (DMA) Protocol & Handshake (Slide 14)

Under Programmed I/O without DMA, every byte requires CPU execution: Time_CPU-IO = N × (T_read + T_write + T_branch). DMA bypasses the processor through a formal 6-step hardware handshake granting direct bus mastership:

+-----------------------------------------------------------------------------------+ ¦ 6-STEP DMA PROTOCOL & BUS ARBITRATION ¦ +-----------------------------------------------------------------------------------+ ¦ Step 1: Peripheral asserts DREQ (DMA Request) to DMA Controller ¦ ¦ Step 2: DMA Controller asserts HOLD (Bus Request) line to the CPU ¦ ¦ Step 3: CPU finishes current machine cycle, tri-states buses, asserts HLDA ¦ ¦ Step 4: DMA Controller takes mastership of System Bus, asserts DACK to Peripheral ¦ ¦ Step 5: High-speed burst data transfer occurs directly between NVMe/Disk and RAM ¦ ¦ Step 6: DMA deasserts HOLD and issues Interrupt (IRQ) signal; CPU resumes process ¦ +-----------------------------------------------------------------------------------+

3.2 Quantitative Comparative Model of the Three DMA Modes (Slide 15)

+-----------------------------------------------------------------------------------+ ¦ 1. BURST MODE: Holds bus for entire transfer (T_stall = N * T_bus). 100% CPU lock.¦ ¦ 2. CYCLE STEALING: Steals 1 bus cycle per byte, alternates with CPU. No starvation.¦ ¦ 3. TRANSPARENT MODE: Steals bus ONLY during CPU internal ALU decode. Zero stall. ¦ +-----------------------------------------------------------------------------------+

3.3 Multicore vs. Multiprocessor Latency & Reliability Model (Slides 16–20)

Memory Access Latency Formulation:
• Multicore (On-chip L3 Cache): T_access = T_L1/L2 + P_miss × T_on-chip_L3 ≈ 0.8 ns to 2.2 ns
• Multiprocessor (Motherboard Bus): T_access = T_L1/L2 + P_miss × (T_egress + T_arbitration + T_trace + T_ctrl) ≈ 15 ns to 65 ns
The mathematical inequality (0.8 ns ≪ 25 ns) proves Slide 20: 'Multicore is faster in executing a single program'.

Reliability Formulation:
• Multicore: P(Failure) = p (single-die point of failure).
• Multiprocessor: P(Complete Failure) = p^k (Socket 2 failure leaves Socket 1 operating, proving Slide 19: 'High fault tolerance').

4. Verification Test Cases & Experimental Results

| DMA Mode | Total Clocks | CPU Work | CPU Stalled | CPU Utilization | Observed Bus Behavior |
| --- | --- | --- | --- | --- | --- |
| Burst Mode | 16 | 0 | 16 | 0% | Full bus lock; CPU in red STALLED state throughout transfer. |
| Cycle Stealing | 32 | 16 | 16 | 50% | Alternating green/red slots on Gantt oscilloscope; no starvation. |
| Transparent | 16 | 16 | 0 | 100% | Zero CPU stalls; DMA sneaks cycles during CPU ALU decode. |

Test Case 2: Test Case 2 (Multicore Rendering): Completed in 1.1s (0.8 ns L3 cache) vs 2.4s on Multiprocessor (84% bus contention serialization).

Test Case 3: Test Case 3 (Fault Injection): Die Overheat crashed all 4 cores (100% outage), whereas Pull Socket 2 preserved Socket 1 running (0% outage), confirming Slide 19.

5. Conclusion & Faculty Sign-Off

The platform successfully validates the concurrency, DMA, and multicore principles of Module 1. Respectfully submitted for academic evaluation.

| Faculty Evaluator: Prof. Shivashish Gour Dept. of Computer Science & Engineering Grade / Marks Awarded: ______________________________ | Evaluation Remarks: _________________________________________ Faculty Signature: _____________________ Date: _____________________________ |
| --- | --- |